# Review Questions

Oct 7, 2026 · 									SATVIK SINGH

## Sensors

1. What is an IMU?

A sensor that measures motion and orientation using an accelerometer (linear acceleration), a gyroscope (rotation rate) and sometimes a magnetometer (magnetic field).

2. 6 DOF vs 9 DOF IMU?

A 6 DOF IMU has an accelerometer and a gyroscope. A 9 DOF IMU adds a magnetometer, which gives an absolute heading reference.

3. 3 DOF of the magnetic component?

The magnetometer measures the Earth's magnetic field along the X, Y and Z axes. Together these give the direction of magnetic north.

4. What is yaw, and how does a 9 DOF IMU reduce it?

Yaw is rotation about the vertical axis, i.e. heading. A gyro-only yaw estimate drifts over time because errors accumulate. A 9 DOF IMU uses the magnetometer's north reference to correct that drift.

5. Encoder vs IMU for the arm?

Encoders are better for joint angles: they are precise, direct and drift-free. An IMU is useful only as a supplement, for example to sense end-effector or base orientation.

## Cameras

6. Why is a stereo camera digital?

Depth comes from matching pixels between two images and computing their offset. That needs discrete pixel data and processing, which only digital output provides.

7. Analog vs digital camera use?

Analog cameras suit low-latency live viewing, such as FPV drones and legacy CCTV. Digital cameras suit anything that is processed, stored or analysed, such as computer vision, robotics and high resolution recording.

## Git

8. git reset vs git revert?

Reset moves the branch back to an earlier commit and rewrites history, so use it only for local work. Revert adds a new commit that undoes an old one and keeps history, so it is safe on shared branches.

9. git pull vs git fetch?

Fetch downloads remote changes without touching your files. Pull is a fetch followed by a merge into your current branch.

10. How many levels in git config?

Three: system (all users), global (current user) and local (one repository). Local overrides global, and global overrides system.

## Linux & Cloud

11. What do you mean by /etc/netplan?

It is the directory in Ubuntu-based Linux distros that holds the network configuration files, written in YAML.

12. What does /etc/netplan do in a Linux distro?

Netplan reads the YAML files there and applies the network settings, such as IP addresses, DHCP or static addressing, DNS and Wi-Fi. It works through a backend like systemd-networkd or NetworkManager. Changes are applied with sudo netplan apply.

13. Is a cloud service a virtual machine? Are all cloud services VMs? Differentiate between them.

No, not all cloud services are VMs. A VM is one kind of cloud service (IaaS, e.g. AWS EC2). Many others are not VMs from the user's side, such as PaaS (Heroku), SaaS (Gmail), serverless functions and cloud storage.

Difference: a VM is a virtualization technology, a full computer emulated in software with its own OS. Cloud is a delivery model: computing resources rented over the internet on demand. Every cloud VM is a cloud service, but not every cloud service is a VM.

14. Advantages of Docker?

It packages an app with its dependencies so it runs the same everywhere. Containers are lighter and faster than VMs, isolated from each other, portable, reproducible from a Dockerfile, and easy to scale.